Scan

The scan feature, which is available on A3 model display radios only, will cause the radio to search through a pre-defined list of channels looking for a condition that will cause the radio to unmute. Whilst searching for unmute conditions, the radio is in 'active scan'; having found the required conditions, the radio will 'land' on the channel. There are two separate, independent scan lists called A and B. One option button can be programmed to start/stop Scan A and another for Scan B.

If channel numbers are entered into a scan list but no option button is programmed to start/stop that scan list, an error message will be sent to the codeplug validation log. Similarly, if an option button is programmed to start/stop a particular scan list but no channels are entered into the list, an error message will be sent to the log.

During active scan, the radio searches through the scan list's channels in the order that the channels appear in the scan list. Channels may be included in the scan list more than once in order to increase the scanning priority of the channel(s) by being scanned more frequently than other channels which are entered only once in the scan list.

The radio supports several scan modes which influence the conditions required for the radio to land on a channel and also the conditions required for the radio to resume active scan. When landed on a channel, the radio enters either sweep mode or listen mode (also known as scan reset mode). Sweep mode allows a fast scan of active channels (like a "scanner"); listen mode allows conversations during scan.

Scan LED

Whilst actively scanning, the Scanning LED indication will be given. When scan lands on a channel, then the Channel Busy LED indication will be given whilst carrier is detected. The Scanning LED indication may be disabled per scan list, in which case no LED indication is given whilst the radio is actively scanning.

Talkback Allowed

Check the identified check box for Scan A and/or Scan B, as appropriate, to enable Talkback Allowed.

With Talkback Allowed enabled, when scan has landed on a channel, transmission will occur on that channel irrespective of the Tx mode setting.

With Talkback Allowed disabled, when scan has landed on a channel, transmission will occur on the channel specified by the Tx mode unless that mode is Last Free or Last Busy, in which case, transmission will occur on the landed channel.

Carrier Squelch Only

When this feature is enabled, the radio ignores the unmute condition required by the channels and instead, scans for the presence of carrier. If carrier is detected the radio will land on the channel and unmute regardless of the receive squelch setting. This allows for a much faster scan, as the radio is not checking for PL/DPL (which takes up to 250ms per channel instead of around 50ms for just checking for carrier).

Include Current Channel

This feature may optionally be enabled per scan list. When enabled, this option will add the current radio channel, i.e. the channel the radio is on at the point at which scan operation is started, to the scan list. If the scan list already contains 16 channels, then the 16th channel will be ignored and the current channel used as the 16th scan channel.

Nuisance Reset

During scanning it is possible to delete channels from the scan list using an option button programmed as "nuisance delete". It is not possible to nuisance delete the priority channel, the last remaining channel in a scan list, or the home channel. All other scan channels, including the 'last free' and 'last busy' channels can be nuisance deleted. It is not possible to attempt nuisance delete when scan has not landed.

If the nuisance reset (on scan exit) option is enabled for a particular scan list, on exiting scan, nuisance deleted channels will be recovered. If the nuisance reset (on scan exit)option is not enabled for a particular scan list, deleted channels can only be recovered by turning the radio off and on again. However, a channel included through the include current channel feature will be lost on scan exit.

Transmit Mode

Where the radio transmits during scan is determined by the scan list's scan Tx mode whether the radio is actively scanning or landed, and if landed, whether talkback is enabled or disabled. The four Tx modes are:

Designated Means the transmit channel is that defined  as 'Designated' for the current scan list.

Home Channel Means the channel in use when scan was started.

Last Free Means the last channel found in the current scan list that was not busy, i.e. the one without carrier. If there are no free channels available then transmit will be disabled until a free channel is found.

Last Busy Means the last channel that scan landed on. When scan is first started, i.e. before it lands on any channels, the transmit channel will default to the last selected channel.

Designated Tx Channel

This enables the RSS to identify a particular channel for the radio to transmit on when the Transmit Mode is set to 'Designated'.

Reset (Hang) Time

Determines how long the radio will remain on the channel after receiving the end of a transmission or after releasing PTT.

Sweep Time

Programmable per scan list and used in the scan 'Sweep Mode'. On entering sweep mode, the radio's sweep timer starts to run and the radio recommences scanning when the unmute condition is lost or when the sweep timer expires.

Sweep mode is disabled for a given scan list by programming a Sweep Time of zero. Note that this configuration is required to make GM350 scan function as per GM300 and GP300, and allow conversations on a PL only channel.

Priority Channel

Any channel may be designated as a priority channel. The programmable priority check interval determines the frequency at which the priority channel is checked both, while the radio is actively scanning other channels and while it is landed on another channel. If priority channel operation is not required, this option is left blank.

If carrier is present on the priority channel then the radio will stay on the channel and look for any other condition required for the radio to unmute. Looking for the unmute condition on anything other then a carrier squelch channel will significantly disrupt the receive audio.

The Priority Channel alert will be sounded when the radio detects the unmute condition on the priority channel. The radio will be in listen mode on landing on the priority channel.

Priority Sample Time

The programmable interval that determines the frequency with which the priority channel is checked  both, while the radio is actively scanning other channels and, while it is landed on another channel.

Priority Channel Alert

The priority channel alert is sounded when the radio detects the unmute condition on the priority channel during scanning. The alert is a high-frequency, interrupted (81 ms on, 97 ms off, 81 ms on)alert tone.

Scan Start Alert

The Start Alert is sounded whenever scan is resumed. The alert is a high-frequency, interrupted (48 ms on, 48 ms off) alert tone, sounded four times.

Scan Lists A and B

Each of the two scan lists may include a maximum of 16 channels. If either list has no entries but an option button is programmed to start that list's scan, an error message will be displayed. If either list has channels included but no option button programmed; again, an error message will be displayed. Unused entries in the scan list should be left blank.