Quick Reference on Programming the Motorola GP-68

- Dial up the channel number that is going to be programmed 1,2,3…20 then:

- Press and hold the LOW button until radio beeps twice. Rotate knob until display shows “Step 5" - this will allow selecting 15KHz, 20KHz or 25KHz channels.  You should only have to do this once.  Once the VFO increment is set you can skip this step.

- Tap the “X” button & enter the RX Frequency (6 digits i.e. 448625 for 448.625) then…

- Press & hold the SQ button until two beeps are heard then…

- Dial up the receive PL tone------tone 00 = carrier squelch, then….

- Quickly press the “B” button & dial up the transmit  PL tone, then…

- Press L arrow button to select offset: plus, minus, zero +- (which gives you a nonstandard offset option), then….

- To write to memory press and hold the lamp button (yellow or green) until two tones are heard and the display shows a “P” then tap it again to enter data into memory.

Handy PL tones/codes:

005=XB=77Hz

007=YZ=82.5Hz

013=1Z=100Hz

014=1A=103.5Hz

015=1B=107.2Hz

017=2A=114.8Hz

020=3A=127.3Hz

021=3B=131.8Hz

026=5A=156.7Hz

084=DPL311

093=DPL411